Supplementary Method Description

Immunohistochemistry. CD171 and CD3 were immunohistochemically detected in formalin-fixed, paraffin-embedded retinoblastoma sections on the VENTANA Benchmark XT automated staining instrument (Ventana Medical Systems, Tucson, AZ, USA) was used according to manufacturer's instructions. Slides were de-paraffinized in EZ prep solution for 30 minutes at 75°C, and antigen retrieval was performed in CC1 solution (Ventana Medical Systems) for 60 minutes at 95°C. Antibodies against CD171 (UJ127.11 clone, diluted 1:100, Thermo Scientific, Waltham, MS, USA) and CD3 (cat #A0452, diluted 1:100, Agilent, Santa Clara, CA, USA) were applied, and developed using the iVIEWTM DAB Detection Kit (cat #760-091, Ventana Medical Systems). Two experienced neuropathologists independently evaluated CD171 staining intensity with concurrence.  Counterstaining of the tumor sections was performed with hematoxylin for 4 minutes. CD3+ lymphocyte (T cell) infiltration into retrospective retinoblastoma tissue sections was assessed using the Stereo Investigator® Software system, including the Stereo Investigator® software (version 11.09 64bit, MBF Bioscience, Williston, VT, USA), an Olympus microscope BX53, the QImaging® camera COLOR 12 BIT and a MAC 6000 stage controller (Ludl Electronic Products Ltd., Hawthorne, NY, USA). The software randomly selected tissue areas, outlined at low magnification (4x objective lens). In this area, the software systematically positioned 400 × 400µm grids. Within these grids, T cells located in the two-dimensional optical dissector (85 × 85µm) were then counted at a higher magnification through the 20× objective lens at regularly pre-determined positions of the grid.